1 Run Server Software

1.1 Fast Manuplation

A. Open Server

Double click to open the Server Software

Program, Default name of the software is “Server_VX.XEn.exe”, “X.X” means the current version, the version in this manual of

our software is V3.0. “Local IP”is the

computer’s IP Address also is Server’s IP 	Fig 1-1

Address.

B. Stop Server

When you open the software, you can see the server has already run. Generally interface as picture. As seen in the picture,

the server will automatically start.  If you need to stop the server, you need unlock it first, the default unlocking key is “123456”.

Fig 1-2

1.2 Advanced Manupaltion

When you stop the Server, you can set some info of software.

1. Server Port: This area you can change the port number used in communicate.

- Connect Port:  Server uses this port to communicate with Intitialize

- Control Port:  Server uses this port to send

control command to IP Terminals

- Audio Play Port:  Server uses this port to send 	Fig 1-3

Audio Signal to IP Terminals

- Audio Capture Port: Server uses this port to accept Audio Signal from IP Terminals5) Manager SW Port:

Warning: If you change the ports in this Interface, you also need to change the corresponding Port in the IP Terminals or the system can’t run properly. So please don’t change this if not necessary.

2. We provide 2 Play Mode for choosing:

- Auto BitRate Timing Mode: server automatically recognizes the bit rate of the MP3 file and then determines the transmission period according to this bit rate, which is determined by the server. This is the so-called public network mode, does not require the terminal to return the request, but the server can only identify the file with a fixed bit rate, does not support the bit rate file, so some files can not be played.

- Player Sync Request Mode: terminal synchronization request, which is used when the LAN or network environment is very good. The terminal will make a request according to the speed of the decoder. The server sends the data after receiving the request, and can play any audio file supported by the decoder.

2.1 OverView

- Double click to open Device Config Tool. Default name of the software is “ConfigTool_VX.X_Client”, “X.X” means the current version, the version in this manual of our software is V3.0.

- As our IP System uses UDP/IP Protocol Stack to send/receive audio/control signal, so the IP Address of each IP Terminal is very important. It should matches the IP of it’s  Superior Router which is depended by the specific working environment. Also Each IP Terminal should know the IP Address of the Server.  So it’s necessary to resize the IP Address and Server IP Address info of each IP Terminal before installing.

- In addition to this,  you can also change the Describe Info such as

Factory/Name/Type/Description etc, and also you can change the UDP Port or Gateway IP Address etc if needed.

Fig 2-1

15

2.2 Connection and Modification

A. Tool Preparation

First you need a computer within Device Configure Tool Software.

B.  Connection

You need to connect computer with IP Terminal through Cables. Directly or with Routers/Switches is okay.

C. Set IP Address

The software uses TCP/IP Protocol Stack to send communicate with IP Terminals, so you need to correctly set the IP Address of your Computer or Routers(If has).  For example,  if you connect directly, then you need to set the IP Address of your computer as “192.168.0.X".

D. Set Terminal

Click connect button , if connection is successful then you will see   at lower left corner. Then type the IP Address of Server and the IP Address of this Terminal you want it be. Finally click   button to set the info into Terminal, you will see  	 and  	 at lower display bar.

2.3 Advanced Tutorial

As seen in 2.1 Overview, you can find that except the IP Address of the Server and the IP Terminal, we can also set many other parameters. We’ll show what others we can set in below.

2.3.1 Describe Info

There’re several column of IP Terminal you can set, the End-User can get this info from the Management Software also. You can also see Fig 3.1 for the details.

- Factory: This column default to set the Factory name or the Installer name.

- Name&Type: Device Type such as Player/Capturer/  Microphone etc.

- Device Description: You can put other info here such as

where you want to put this

device. 	Fig 2.2

Warning: You can see also Device ID and Mac Addr Column in Fig 2.2, Device ID is the unique identification number of the terminal, the Software uses this to identify terminals and manage their IP Address and Tasks. If 2 Terminals have 1 ID then the software can’t run. The ID is set before leaving factory so usually you don’t need to change it. So the Client Version of Config software can’t change this. If you do need to change this, please contact your installer.

2.3.2 IP Address and Port	A. IP of Terminal

The configuration is similar to the configuration of Computer, but now our version(3.0) don’t support

DHCP service so it need to set to match the upper Router.

B. IP of Server

IP Address is same to the IP Address of the Computer that run Server, you can see it also when you run the server at the interface.

Warning: You can see 4 Ports number also in this area, IP Terminals will use these 4 ports to communicate with the Server, so this 4 columns need to be same with the 4 columns in Fig 1.2, which means you need to change them in same time. You only want to change this when the

Fig 2.3 	computer has software using one or more of the 4 Default ports and you can’t change them.

2.3.3 Fast Save & Batch Operation

- Save/Load

You can set the current data of the columns into a file or Backup them in Software, you can get it by click   or  .

- Batch Operation

If you click the Check Box , then you can click   Button to download data into

Terminals, and then the IP Address of Device Area will increase by 1, it’s simple if you need configure many terminals.

Fig 2.4

15

3 Initialize Management Software

3.1 Login

A. Open Software

Click the Management Software Program, Default name of the software is

“Manager_VX.xEn”,“X.X”means the current version, the version in this manual of our software is V3.0.

B. Login

When you open the file, click “Log in”button on the left upper corner. You’ll see Login Interface as Fig 3-1.

Please type the IP Address of the Server in corresponding column, type the password and click “OK” button. You can click Memorize Check box to memorize the IP Address of last login.

Fig 3-1

Fig 3-2

15

3 Initialize Management Software

3.2 Registration

When you login Management Software successfully, you can see interface as Fig 3.2, as seen in the picture, you can find   on the lower display bar. It’s because you need to enter a registration code from your Service Provider.

A. Enter the system interface

You will find a Tool Bar on the top of the Interface as Fig 3.4, click the System button and then you’ll enter system interface.

Fig 3-3

Fig 3-4

When you open you can see “NA” in the Client ID/User SW Manage Code Columns, it means the software haven’t been registered yet. Click Authorize Button in Fig 3.3, you can see new Column appears as Fig 3-5. Enter the Configuration Code get from your

service provider and click Set button. you can see as Fig 3-6, and also see   at lower display bar.

Fig 3-5

Fig 3-6